	FY 2019 NPP 14 – Aug – In Flt Wx Resources – REV. Orig 15 August 2019



In-flight Weather Resources
2019/08-15-172(I)PP
Abstract:   Lasting 10 to 20 minutes, this presentation acquaints the audience with some currently available weather information resources that are available to pilots while in flight.  Because the number and availablility of in-flight weather information resources is rapidly increasing, we are unable to list them all.  This program covers a sampling of resources that were available as of August 2019.
Format:  Information Briefing  - Power Point presentation 

Required Personnel – FAASTeam Program Manager or designated FAASTeam Rep (s)
Optional Personnel – CFIs and DPEs who can speak on the availability and use of in-flight weather information.
AFS 850 Support:

In addition to this guidance document, a Power Point presentation that supports the program is provided. FPMs and presenters are encouraged to customize this presentation to reflect each individual program.  
Appendix I – Equipment and Staging

Equipment:

· Projection Screen & Video Projector suitable for expected audience
· Remote computer/projector control available at lectern or presenter location
· In lieu of remote – detail a Rep to computer/projector control.
· Presentation Computer
· Note:  It is strongly suggested that the entire program reside on this computer.
· Back up Projector/Computer/Media as available.
· PA system suitable for expected audience
· Microphones for Moderator and Panel
· Optional Microphone (s) for audience
· Lectern (optional) 
Staging: 

· Arrange the projection screen for maximum visibility from the audience.
· Equip with PA microphones
· Place Lectern to one side of screen.  This will be used by presenters and moderator
	Slides
	Script

	[image: image1.png]The National FAA
Safety Team Presents

Topic of the Month — August
In-flight Weather Resources
Prasentd o

8y
Date:

Produced by AFS-850
The FAA Safety Team (FAASTeam)

Federal Aviation
Administration





	Slide 1

2020/07-01-194(I)PP Original Author: John Steuernagle 01/30/2019;  POC Kevin Clover AFS-850 Operations Lead, Office (562-888-2020) revised by AJR B-6 07/01/2020.

Presentation Note:  This is the title slide for In-flight Weather Resources.
· Script -  We have included a script of suggested dialog with most slides.  The script will always appear in a non-italic font.  Presenters may read the script or modify it to suit their own presentation style.  See template slides 5 and 6  for examples of a slides with script.

· Presentation Instructions - (stage direction and  presentation suggestions) will be preceded by a  Bold header: the instructions themselves will be in Italic fonts.  See slides 2, for an example of slides with Presentation Instructions only.

· Program control instructions - will be in bold fonts and look like this:  (Click) for building information within a slide;  or this:  (Next Slide) for slide advance.

· Background information - Some slides may contain background information that supports the concepts presented in the program.  
Background information will always appear last and will be preceded by a bold  Background: identification.

The production team hope you and your audience will enjoy the show.   Break a leg!  
(Next Slide)

	[image: image2.png]Welcome

* Exits

* Restrooms

+ Emergency Evacuation

* Breaks

+ Phones & pagers to silent or off
+ Sponsor Acknowledgment

« Other information





	Slide 2
Presentation Note: Here’s where you can discuss venue logistics, acknowledge sponsors, and deliver other information you want your audience to know in the beginning.  
You can add slides after this one to fit your situation. (Next Slide) 

	[image: image3.png]Overview

+ *GAJSC Safety Enhancement — Weather Technology

+ Pre-flight weather briefing is
assumed

«+ In-flight weather resources
— Not a comprehensive list

~ Sample of resources available
today

Gl raon i S cormtes





	Slide 3
The General Aviation Joint Steering Committee is a government/industry group that looks at GA mishap experience and makes recommendations (they call them safety enhancements) for safety improvement.  This program addresses their recommendation that GA pilots be apprised of advances in weather technology that can enhance the safety of flight.

In this program we’ll assume you got a thorough pre-flight weather briefing and discuss only those weather information resources that are available to general aviation pilots in flight.   Technology is advancing so quickly that we can’t claim our list of resources is at all comprehensive but rather a sampling of what’s available to pilots today.

 Presentation Note: If you’ll be discussing additional items, add them to this list. 
(Next Slide) 

	[image: image4.png]AC 00-45H Aviation Weather Services

+ Comprehensive list of FAA, NWS services
and products

~ Includes FIS-B (Flight Information Service-
Broadcast) products
* Available through ADS-B In data link

* Additional information resources will be
cited at the end of this presentation.





	Slide 4
Much of the information presented here is taken from FAA Advisory Circular 00-45H – Aviation Weather Services.  This publication contains a comprehensive list of weather services and products from the National Weather Service (NWS) and FAA including a list of Flight Information Service-Broadcast (FIS-B) products that are available through ADS-B-In data link.  (Click)
 We’ll list additional information resources at the end of this presentation. 
(Next Slide) 

	[image: image5.png]AC 91-92 Pilot’s Guide to a Preflight Briefing

Tentative Release 2021

+ Encourage pilots to conduct regulatory compliant preflight self-briefings

+ Show pilots how to conduct a regulatory compliant prefiight seif-briefing
using automated resources

« Assist pilots in complying with Title 14 of the Code of Federal Regulations
(14 CFR) part 91, §§ 91.103 that states each pilot in command shall
become familiar with all available information concerning their flight before
departure

+ Replaces FAA publications:
— General Aviation Pilot's Guide to Preflight Weather Planning, Weather self-Briefings,

and Weather Decision Making

— How to Obtain a Good Weather Briefing





	Slide 5

AC 91-92, scheduled for release in 202 will offer valuable information on the conduct of self briefings.

AC 91-92 ) note  Encourage pilots to conduct regulatory compliant preflight self-briefings
- Show pilots how to conduct a regulatory compliant preflight self-briefing using automated resources
- Assist pilots in complying with Title 14 of the Code of Federal Regulations (14 CFR) part 91, §§ 91.103 that states each pilot in command shall become familiar with all available information concerning their flight before departure
- Will replaces FAA publications:
- General Aviation Pilot’s Guide to Preflight Weather Planning, Weather self-Briefings, and Weather Decision Making
- How to Obtain a Good Weather Briefing
(Next Slide)

	[image: image6.png]Weather analysis and decision making are
big parts of our job.

P





	Slide 6
Weather analysis and decision making are big parts of every pilot’s job.  We know the weather at our departure airport but en route weather is less certain.  (Click)
And of course we need to know if weather conditions will permit a landing at our destination.

 calculate performance based on the weight you expect to be while flying.  
(Next Slide) 

	[image: image7.png]Preflight to tie down

* Perceive

— Gather weather information
* Process

~ Analyze weather information
* Perform

~ Make and act upon
operational decisions

Perceive  Process

Perform




	Slide 7
Weather information gathering, analysis, and decision making begin with the preflight and continue throughout.  While en route we need to continually:  (Next Slide)
Perceive present weather conditions and gather additional weather information from various sources. (Click)
Process weather information by comparing present weather with forecasts and by relating present and expected weather conditions to aircraft and pilot performance capabilities. (Click)
Perform by making operational decisions and acting upon them. (Click)
This process continues throughout the flight.  Our perception of the results of our performance decisions further informs and refines our analysis.

 (Next Slide) 

	[image: image8.png]AC 91-92 Pilot’s Guide to a Preflight Briefing

Tentative Release 2021

+ Encourage pilots to conduct regulatory compliant preflight self-briefings

+ Show pilots how to conduct a regulatory compliant prefiight seif-briefing
using automated resources

« Assist pilots in complying with Title 14 of the Code of Federal Regulations
(14 CFR) part 91, §§ 91.103 that states each pilot in command shall
become familiar with all available information concerning their flight before
departure

+ Replaces FAA publications:
— General Aviation Pilot's Guide to Preflight Weather Planning, Weather self-Briefings,

and Weather Decision Making

— How to Obtain a Good Weather Briefing





	Slide 8
Regulations require that, during our pre-flight planning, we gather all pertinent information with regard to each flight.

We can use automated weather information resources to gather weather information without  contacting Flight Service

Pilot can still contact Flight Service but they are highly encouraged to conduct a self-briefing prior to the call.  That way they’ll have 
a good idea and understanding of weather conditions before speaking with a briefer.

(Next Slide)

	[image: image9.png]Pre-flight Know Before You Go

+ Wieather advisores (SIGMETs, AIRWET, Convectve SIGNETs, Certer Weather
‘Advisories, Aviaton Watch Noification Messages)

- NOTAMs (aiportumway closures, TFRs, etc.)

- IFR condilons, low-evel windshear, thunderstors, reported icing,frontal zones.

2) Synopsis. + Weather systems andlor air masses

1) Adverse Conditions

- Curent abservations (¢.9., METARS, PIREPs) for departure, en route, and destination

3) Current Conditions - Satelite and radar imagery

4) Forecast Conditions. - Forecast nformation (departure, en route, and destination)

= Winds slof forecas (ierpoiate beteen Ievels and staions)
AL - Tomperature at proposed attude

6) Notices to Alrmen (NOTAM) |- Departure, en route, and destination

7) Restricted or Special Use |- Profibited Areas P40, P-56, and the Special Fight Rules Avea (SFRA) for
Airspace ‘Washington, DC

8) ATC Delays AT delays andior flow coniro advisories

) e





	Slide 9
· Flight Service uses this general list of elements for providing Standard Weather Briefings.  

· It is recommended that this list be used when conducting a preflight self-briefing using automated resources.  

· Using this list of elements when conducting a preflight self-briefing regardless of the automated resource will assist a pilot in complying with Title 14 of the Code of Federal Regulations (14 CFR) part 91

 HYPERLINK "https://www.ecfr.gov/cgi-bin/text-idx?SID=8404b1a25952bc103e6743e6fc72f700&amp;mc=true&amp;node=pt14.2.91&amp;rgn=div5" , §§ 91.103.
· There are also interactive graphics (eg GFA or 1800wxbrief.com Interactive map) available that depict some of these elements to assist in understanding what information is applicate to the route of flight. 

(Next Slide)

	[image: image10.png]Sample Pre-flight Self-Briefing Resources

1800wxbrief.com
avcams.faa.gov

Leidos Flight Service — FAA Contract Vendor

| Alaskaand Canada

aviationweather.gov

NOAA/Government Web Site for aviation weather

Fly.faa.goviflyfaalusmap.jsp
nhc.noaa.gov

FAAFlight Delay Information
National Hurricane Center (HNC)

notams.aim.faa.govinotamSearch
spc.noaa.gov

Federal NOTAM System (FNS)
NOAA Storm Prediction Center

ssd.noaa.gov/VAAC/vaac.html

Volcanic Ash Advisory Centers (VAAC)

sua.faa.gov | Special Use Airspace

tfr.faa.gov/tfr2/list.htmi Temporary Flight Restrictions

weather.gov National Weather Service Forecast Office
weather.goviaawu Alaska Aviation Weather Unit (AAWU)
weather.gov/hfo National Weather Service Forecast Office Honolulu, HI

wpc.ncep.noaa.gov

Weather Prediction Center





	Slide 10

Listed are government resources, there are also third-party automated resources that may be used for conducting pre-flight self-briefings

FAQ on using automated preflight resources.  You may get some of the questions listed below.  If so refer to the answer listed.

Q1: Does my self-briefing need to be recorded or documented by the automation sites(s) I use?

A1: There is no requirement for a self-brief to be recorded

If you prefer to have your self-briefing preparation be recorded, consider invoking the Standard Briefing functionality on 1800wxbrief.com; third-party applications may also offer a recorded briefing functionality

Q2: I like the Graphical tools (GFA, Interactive Maps) when conducting preflight planning. Do I still need to use the “Standard Briefing” functionality on the planning website or app that provides a full textual briefing?

A2: Full textual (aka Standard Briefing) functionality is not required to be invoked  

The goal is to conduct a comprehensive preflight self-briefing; if you have done this (i.e., covered all the checklist items) using the graphical tools, you do not need to invoke a full textual briefing 

However, the Standard Briefing functionality is a good idea as a double check that you have not missed something in your own workflow

Q3: If I conduct a self-brief and still decide to call Flight Service, will they know what I have done online?

A3: YES, if you have used 1800wxbrief.com and requested a standard briefing

YES, if you have used a third-party application that links to your 1800wxbrief.com account and uses an approved standard briefing product (check with your particular third-party vendor)

NO, if you have used just the graphical functionality on 1800wxbrief.com, AWC, or third-party sites

If you plan to call Flight Service, then invoking the Standard Briefing functionality from 1800wxbrief.com (or a third-party application that shares your 1800wxbrief.com credentials) will allow the Specialist to see your briefing details and save you time

(Next Slide)

	[image: image11.png]In-flight Weather Sources
« Flight Service (FSS) en route weather

« Frequencies depicted on VOR data block
+ FSS transmits & receives on 122.2 (VHF) and 255.4 (UHF)
+ FSS transmits and receives on 122.6 of receives on 122.1 & transmils on 108.8 (VOR)
189y P -

A,
<%

N i o





	Slide 11
Let’s begin with FAA Flight Service.  FSS stations provide en route weather via voice communication in a couple of different ways.  In this example, aircraft flying near the Kankakee VOR can contact Flight Service on 122.2 Mhz.  Aircraft equipped with UHF Comm radios can contact FSS on 255.4 Mhz.  (Click)
Here Flight Service can transmit and receive on 122.6 Mhz. or they can receive on 122.1 Mhz. and transmit over the Mansfield VOR on 108.8 Mhz.  If you choose the VOR communication option, be sure to enable voice on your VOR receiver and adjust the audio volume so you can hear the FSS transmissions.

 (Next Slide) 

	[image: image12.png]In-flight Weather Service Products — Voice or Text

Near Real Time Forecasts
or older old news

METARS - Surface Obsenvations Area Forecasts - FAs

SPECIS - Special METAR Reports Terminal Aetodrome Forecasts - TAFs

SIGMETs (WSs)- Non-convective weather Winds & Temperatures Alof

hazards o al acraft \
1o ved

Convective SIGMETs (WSTs) - Convective weather forecast®t® i ine

hazarcs to al acraft compar® % s t0 8°

ARMETS (Whs) - Weather hazarcs o ghtaicrant e OBSETCL oicture:

PIREPs - Pilot reports of light conditions. pest WE

NOTAMSs — Notices to Airmen





	Slide 12
1. Here are some of the products available for en route access.  You may receive this information via radio or via data link.  More about data link later.  

2. Whenever you access weather information it’s important to know how old that information is.  Some of the products in the left hand column could be more than an hour old when you receive them.  Others may be quite recent.  That’s why it’s important to note the times when observations were taken and/or the product validity time span.  

3. Products in the right hand column will be older than those in the left.  (Click)
4. Forecasts work best when you compare them with more current weather observations.  That way you can see if weather is developing earlier or later than the forecast schedule.
5. PIREPs – Pilots are the best resource for reporting inflight weather conditions and confirming forecasts are correct or not. 
(Next Slide)

	[image: image13.png]In-flight Weather Sources
azardot ervice (HIWAS)
« Flight Information Services-Broadcast (FIS-B)





	Slide 13
The letter H in a VOR data block indicates a couple of things:

·  the ability to receive HIWAS (Hazardous in-flight Weather Advisory broadcasts.  As the name implies, these were pre-recorded advisories of hazardous weather conditions.  (Click)
· It also indicates that you’re consulting an out dated chart because HIWAS was decommissioned in January of 2020. (Click)
Also in January most aircraft are required to be equipped with ADS-B out.  For those of us who are equipped with ADS-B In, FAA is providing an impressive set of weather information products collectively known as Flight Information Services – Broadcast or FIS-B.

 (Next Slide)

	[image: image14.png]ADS-B - In Applications

+ Automatic Dependent Surveillance — Rebroadcast (ADS-R)
+ Traffic Information Services - Broadcast (TIS-B)
+ Flight Information Services - Broadcast (FIS-B)





	Slide 14
Although ADS-B In equipment is not required, there are some compelling reasons why pilots are considering installing it.  As you can see from the coverage map, en route ADS-B In service is available throughout the country.  With ADS-B In you get a suite of traffic, weather, and flight information products.  Let’s take a look at how it works.  

(Next Slide) 

	[image: image15.png]ADS-R
« UAT ADS-B In aircraft
receive UAT ADS-B out
traffic directly

* ADS-R relays ADS-B Out
Information to ADS-B In-
equipped aircraft





	Slide 15
UAT equipped ADS-B In aircraft receive traffic information directly from UAT ADS-B out aircraft. (Click)
ADS-R rebroadcasts traffic information derived from UAT and 1090 Extended Squitter positions so that even though the two systems don’t exchange information directly, traffic information is available to both. 

There’s also a way for ADS-B in users to see aircraft that are not ADS-B Out equipped.

(Next Slide) 

	[image: image16.png]TIS-B

* Available to ADS-B In
aircraft at and below
24,000 feet

« Positional data

collected from all

aircraft is transmitted
to ADS-B In aircraft





	Slide 16
The ADS-B system transmits TIS-B data on both links to equipped aircraft flying within coverage.  TIS-B uses data from ADS-B, radar, Wide Area Multilateration (WAM), and surface multilateration systems like ASDE-X to create accurate near-real-time position reports.

In order to receive TIS-B information you must be ADS-B In equipped, within coverage of an ADS-B ground station and an FAA radar or multilateration system.  

Background:  Multilateration systems are employed to provide surveillance outside of radar coverage.  

· Wide Area Multilateration (WAM) is comprised of a network of relatively small sensors that can be deployed in areas that are problematic for radar installations.  WAM capabilities an be combined within a set of ADS-B ground stations.

· Surface Multilateration consists of sensors deployed on airports.  The data provided by these sensors is integrated with airport surveillance radar and ADS-B sensor data to create an accurate picture of ground traffic on the airport.  The equipment and sensors is collectively known as Airport Surface Detection Equipment (ASDE-X)

(Next Slide)

	[image: image17.png]Caution

+ ADS-B traffic picture may
not be complete

+ Good adjunct to visual scan but...

+ We still need to look
outside for traffic.





	Slide 17
ADS-B In is not guaranteed to display all traffic in a given area.  It’s a great situational awareness and collision avoidance tool but it’s also a compelling distraction.  We still need to look outside our windows.  

(Next Slide)

	[image: image18.png]ilable to ADS-B UAT
equipped aircraft
System broadcasts
aeronautical information
products from the FAA
and weather products
from the National
Weather Service





	Slide 18
FIS-B information is available to UAT (Universal Access Transceiver) equipped aircraft only.  The system broadcasts aeronautical information products from the FAA and weather products from the National Weather Service.

(Next Slide)

	[image: image19.png]FIS-B Products

AIRMETs TAFs

Convective SIGMETs. Amended TAFs

SIGMETS Winds & Temperature Aloft
METARS Lightning

SPECIs Turbulence

National NEXRAD leing

Reglonal NEXRAD Cloud Tops

D-NOTAMS Graphical AIRMETs
FDC-NOTAMs Center Weather Advisories
PIREPS. TIS-B Service Status

‘Special Use Alrspace (SUA) Status





	Slide 19
Here’s a list of text and graphical FIS-B products – available free of charge to ADS-B In users.  As you can see – it’s quite extensive and more products may be added in the future.  

(Next Slide)

	[image: image20.png]In-flight Weather Sources

+ Automated Terminal Information Service (ATIS)
o @@é‘v 2593 » C ok





	Slide 20
We’re all familiar with Automated Terminal Information Service available at most towered airports.   ATIS provides basic weather and runway information but what about non-towered airports?

(Next Slide)

	[image: image21.png]In-flight Weather Sources
+ Automated Surface Observing System (ASOS)
+ Automated Weather Observing System (AWOS)

G CIRCUS

NI A 4
b





	Slide 21
Many smaller airports are served by ASOS or AWOS automated weather observing systems.  In fact, most of the weather data used to produce aviation weather reports and forecasts is now gathered by automated systems.  These systems are constantly updating weather observations and averaging the data to give an accurate picture of present conditions.  There are about one thousand ASOS and even more AWOS systems deployed throughout the United States.

(Next Slide)

	[image: image22.png]ASOS Information

METAR Element Information Provided

Wind Direction, Speed, & Character Tens of degrees ~ Knots, Gusts

Visibility Up to & including 10 statute miles

Runway Visual Range (RVR) At selected sites

Basic present weather Type and Intensity

Obstructions to vision Fog, mist, haze, & freezing fog

Sky condition Cloud height and amount to 12,000 Ft. AGL
CLR, FEW, SCT, BKN, OVC

Amblent & Dew point Temperatures Degrees Celsius

Pressure Altimeter Setting In. Hg.

Remarks Automated, Manual, & Plain Language -

depending on service level




	Slide 22
Here’s a chart of what’s available from an ASOS installation.  At some locations information may be augmented by human observers.

Presentation note:  Take a minute or so to discuss the information provided by ASOS, then:
(Next Slide)

	[image: image23.png]AWOS Information

AWOS Type
AWOS-A

AWOS-AV
AWOS-1

AWOS-2
AWOS-3

AWOS-3P
AWOS3PT
AWOS-3T
AWOS4

Information Provided
Altimeter Setting
Altimeter Setting & Visibility

Altimeter Setting, Wind speed & direction,
Temperature, Dew Point, & Density Altitude

AWOS-1 plus Visibility
AWOS-2 plus Cloud & Celling Data

AWOS-3 plus Precipitation Discriminator
AWOS-3P plus ThunderstormiL ightning
AWOS-3 plus ThunderstormiLightning

AWOS-3 plus precipitation type and
accumulation, freezing, thunderstorm, &
runway surface information




	Slide 23
As you can see there is a wide range of information available from AWOS depending on the type of equipment installed.  (Click)
Most of the AWOS installations are a variation of AWOS-3 so you’ll have Altimeter Setting, Wind, Temperature (older installations may report in degrees Fahrenheit), Dew Point, Density Altitude, Cloud and Ceiling data.  Newer installations may add precipitation type and thunderstorm/lightning information.

(Next Slide) 

	[image: image24.png]Airborne Weather Radar





	Slide 24
For those of us who can afford it, Airborne Weather Radar is a very useful information source.  It does require interpretation though and there are some limitations to consider. 

 (Next Slide) 

	[image: image25.png]Airborne Weather Radar

* Real Time

+ May not show all
precipitation

* Good for tactical
avoidance

* New users should seek
instruction





	Slide 25
Airborne radar looks at precipitation and pilots know that the heavier the precipitation – the greater the associated turbulence will be.  Because radar returns are immediate it’s a good choice for tactical weather avoidance.  Heavy precipitation can mask additional precipitation behind it so it’s best to find wide clear areas so you can get a good picture of what’s beyond the radar return. 

If you’re a new radar user be sure to get some instruction on operation and interpretation before you base tactical decisions on radar information.

(Next Slide)

	[image: image26.png]Strike Finder & Storm Scope

* Real Time

* Doesn’t show
precipitation

+ Good for tactical
avoidance

« Instruction

recommended for
new users





	Slide 26
Another real time solution is lightning detection equipment.  Although it doesn’t show precipitation it does indicate lightning strikes that accompany convective weather.  Some installations integrate radar and storm scope information on a multi-function display.

As with radar, instruction is highly recommended for new users.

Next Slide)

	[image: image27.png]Portable ADS-B Receivers





	Slide 27
If you don’t want to equip with a panel-mounted ADS-B In, there are many portable ADS-B Receivers to choose from.  These receivers can transmit text and graphics to a variety of portable devices and displays.  One caution here – you’ll need to have sufficient battery capacity or auxiliary power available to power receivers and displays for the planned duration of your flight and any delays or diversions that may occur.

(Next Slide)

	[image: image28.png]XM Weather

* Near Real Time
— 2.5 Minute update
~ Look for time stamp

+ Good for strategic
planning





	Slide 28
XM Radio is a subscription service that provides test, graphical, and audio information to en route users.  XM Weather imagery is processed in near real time – usually at two and a half-minute intervals.  But the imagery information could be older so, as with any weather information, be sure to check the time that the graphic data was collected. 

(Next Slide)

	[image: image29.png]Air Traffic Control

+ Limited ability and time to
forward weather information
* You can learn a lot by
just listening. =
* Don’t wait until the last minute §
to make diversion requests.





	Slide 29
Air traffic controllers are another in-flight weather information resource.  They do have the ability to show weather on their displays but that capability is limited and when you most need information they may be

too busy – dealing with re routing requests – to provide it.

You can learn a lot by monitoring air traffic control frequencies though.  You’ll hear what pilots are requesting and what ATC can provide.  If you make your weather decisions with that in mind your requests are more likely to be granted.

Once again – don’t wait till the last minute to request a diversion.  ATC needs some time to process your request.

(Next Slide)

	[image: image30.png]Cautions & Tips

« Don’t fixate on the equipment

~ Cockpit displays don't tell the whole story
+ Westil have to look outside

+ Understand what the displays tell you... and what they don’t.
~ You may not see all the traffic in your area
~ or all the weather ahead

+ Make weather avoidance decisions early
~ Don't wait fill you're close to choose a route

~ Refine your decisions as more information
becomes available





	Slide 30
In-cockpit information displays can be compelling distractions.  Don’t fixate on the box to the exclusion of other flight management tasks.  Cockpit displays – no matter how sophisticated and comprehensive they may be – don’t tell the whole story.  A big part of our job is still looking out the window. 

It’s important to thoroughly understand what the displays are telling you, and just as important, what they aren’t.  Displays may not depict all the traffic in your area or all the weather ahead.  

Generally speaking it’s best to make in-flight decisions early rather than waiting until the last minute to act.  Landing to refuel is much less stressful if you have an hour or so of fuel remaining.  Likewise rerouting or diversion is easier if  you have some maneuvering room and time to select the best alternative.  

By continuous reference to all weather resources available to you in the cockpit, you’ll have a comprehensive picture of the weather and any decisions will be well informed. 

(Next Slide)

	[image: image31.png]Proficiency training works!

+ Sports
+ Medicine




	Slide 31
One more thing -  WINGS participation also means chancCan you imagine how well  professional athletes would perform if they didn’t practice between games or stay in shape during the off season? (Click)
Would you choose to be treated by doctors who had no continuing education since graduating from medical school?  (Click)
Or how about a professional pilot flight crew who never train for emergencies? 

Pros know that proficiency is not a destination but rather a journey that never ends.  

 

Regular training keeps the them at peak performance every time they take to the air.

(Next Slide)

	[image: image32.png]Proficiency Training Works

Increases confidence
Increases comfort
Expands horizons

Keeps us safe

Earning any WINGS phase

qualifies for a Flight Review! !





	Slide 32
Proficiency training works for General Aviation pilots too.  Pilots who participate in the FAA WINGS Pilot Proficiency program fly with more confidence.  They and their passengers are comfortable in the air.  And proficiency training can expand our horizons by exploring the operational capabilities of our present aircraft or introducing us to more complex and capable planes as well as more challenging destinations.  

Most importantly - proficiency training keeps us safe.  (Click)
And pilots who earn WINGS phases also qualify for a flight review.

(Next Slide)

	[image: image33.png]Why WINGS?

* Proficient Pilots are:
~ Confident
— Capable
— Safe

+ WINGS will keep you
on top of your game





	Slide 33
So why WINGS?  Well proficiency is key to success in almost every thing worth doing – especially flying.  Proficient pilots are confident, capable, and safe.  

WINGS is a proficiency training system specifically designed for general aviation pilots and, regular participation will keep you on top of your flying game. (Next Slide)

	[image: image34.png]WINGS Also Means Chances to WIN $$$

Introducing the WINGS Industry Advisory Committee

WINGS

Industry Advisory Comittee,





	Slide 34
One more thing -  WINGS participation also means chances to win cash!  (Click)  
The WINGS Industry Advisory Committee is a coalition of FAASTeam Industry Partners, associations and devoted individuals committed to increasing awareness and participation in the WINGS program. And they believe it WINGS enough to offer chances to win cash prizes for completion or validation of WINGS phases!

 (Next Slide)

	[image: image35.png]The Paul & Fran Burger 2019
WINGS $10,000 Sweepstakes

Complete WINGS Phases and WIN Cash Awards!

www.mywingsiniative.or





	Slide 35
Here are a few references for additional information:  

I’ll leave this slide on screen while I take some questions from the audience.

Presentation note:  Take questions from the audience while they copy information from the screen.  Then:
(Next Slide)

	[image: image36.png]References

+ AC 00-45H Aviation Weather Services
~ https://www.faa.govidocumentlibrary/media/advisory_circular/ac_00-45h.pdf

+ AC 00-63A Use of Flight Deck Displays of Digital Weather and
Aeronautical Information

— https://www.faa.gov/documentLibrary/media/Advisory_Circular/AC_00-63A.pdf

+ AC 90-114B Automatic Dependent Surveillance-Broadcast
Operations
— https://www.faa.gov/documentL ibrary/media/Advisory_Circular/AC_90-114B.pdf





	Slide 36
Here are a few references for additional information:  

I’ll leave this slide on screen while I take some questions from the audience.

AC 90-114B Automatic Dependent Surveillance-Broadcast Operations

AC 00-63A Use of Flight Deck Displays of Digital Weather and Aeronautical Information  (This AC has a Change Revision CH2 coming out very shortly)  This AC addresses both the Federal Aviation Administration (FAA) FIS–Broadcast (FIS-B) provided through the Automatic Dependent Surveillance–Broadcast (ADS-B) Universal Access Transceiver (UAT) network and non-FAA FIS systems provided through commercial data link services.

Presentation note:  Take questions from the audience while they copy information from the screen.  Then:
(Next Slide)

	[image: image37.png]References
+ AC 00-45H Aviation Weather Services

* Aeronautical Information Manual

— Chapter 7 - Safety of Flight
- Secton - eteorsogy

« FAA ADS-B Information

+ Automated Surface Observing Systems





	Slide 37
Here are a few references for additional information:  

I’ll leave this slide on screen while I take some questions from the audience.

Presentation note:  Take questions from the audience while they copy information from the screen.  Then:
(Next Slide)

	[image: image38.png]Questions?





	Slide 38
Presentation Note:   You may wish to provide your contact information and main FSDO phone number here. You can also add WINGSPro contact information. 
Modify with your information or leave blank.   
(Next Slide)

	[image: image39.png]Thank you for attending

* You are vital members of our GA safety
community





	Slide 39
Presentation Note:   You may wish to provide your contact information and main FSDO phone number here. You can also add WINGSPro contact information. 
Modify with your information or leave blank.   
(Next Slide)

	[image: image40.png]The National FAA
Safety Team Presents

Topic of the Month — August
In-flight Weather Resources
Prasentd o

8y
Date:

Produced by AFS-850
The FAA Safety Team (FAASTeam)

Federal Aviation
Administration





	Slide 40

(The End)


2

